MSRS Report Format Documentation

Operating Reserve Deviation Summary

Version 8

Revision History

| DATE | Revision | Description |
| --- | --- | --- |
| 10/01/2007 | 1 | Initial Distribution |
| 1/14/2008 | 2 | Changed the XML column name for the DA Operating Reserve Imports column from DA_OPERATING_IMPORTS to DA_OPRES_IMPORTS |
| 6/30/2008 | 3 | Changed the XML column name for the DA Internal Bilateral Sales column from DA_INTERNAL_SALES to DA_INTERNAL_BILATERAL_SALES |
| 10/15/2008 | 4 | Updates for Balancing Operating Reserve Changes. Added the following columns: Bal Operating Reserve Location Name Bal Operating Reserve Location ID Location Type Bal Operating Reserve Region Name Locational Total Deviation (MWh) |
| 1/14/2013 | 5 | Updates to column numbers to match report standards |
| 3/28/2013 | 6 | Updates to section 9 column numbers to match report |
| 8/5/2015 | 7 | Added Load Reconciliation Energy (3000.77) column. Updated supporting calculations. |
| 2/21/2020 | 8 | Removed all references to eSchedule(s) and/or eSchedule(s) reports. |

# Report

MSRS Report Name: Operating Reserve Deviation Summary

Report short name for User Interface: Operating Reserve Deviation Summary

Download File Name Abbreviation: ORDev

Data Granularity:  Hourly

Frequency: Updated daily

Range Displayed on Report:  Start Date through End Date

# Supported Billing Line Items

- Balancing Operating Reserve Charge (1375)

# Report Content Summary

This report displays the customer account’s hourly generator deviation and also provides the hourly values for the components used in calculating withdrawal and injection deviation MWh which are used in calculating the Balancing Operating Reserve Charge.  These values are displayed by balancing operating reserve location.

# Summary of Changes and Special Logic

- When this report is selected for a trade date of 12/1/2008 or beyond, the Bal Operating Reserve Location Name, Bal Operating Reserve Location ID, Location Type, Bal Operating Reserve Region Name and Locational Total Deviation columns that were added to accommodate the operating reserve accounting methodology changes will appear in this report.  When this report is selected for a trade date of 11/31/2008 or prior, the previous format of the report (which does not contain these new columns) will appear.  Please note that due to these different report formats, requests for this report will not be allowed to span the cutover trade date of 12/1/2008.

- The date range total row will only appear in the online version of the report.  It will not appear in the CSV and XML versions of the report.

- The Balancing Operating Reserve Location ID and Name displayed on this report will be the PNode ID and Name of the zone, hub or interface PNode at which the deviations occurred.

- The Balancing Operating Reserve Location Type displays the type of PNode at which the deviations occurred.

- The Balancing Operating Reserve Region Name will display the name of the balancing operating reserve region in which the deviation location falls.  ‘RTO’ will be used to denote the name of non-region deviations (deviations at locations that do not fall within either the East or West balancing operating reserve regions).

# Report Columns

The following columns will appear in the body of the report:

| Online and CSV Column Name | XML Column Name | Column Number | Data Type |
| --- | --- | --- | --- |
| Customer ID | CUSTOMER_ID | 4000.01 | INTEGER |
| Customer Code | CUSTOMER_CODE | 4000.02 | VARCHAR2(6) |
| EPT Hour Ending | EPT_HOUR_ENDING | 4000.05 | VARCHAR2(40) mm/dd/yyyy HH24 format (Displays first hour of the day as hour 1 and last hour of the day as hour 24) |
| GMT Hour Ending | GMT_HOUR_ENDING | 4000.06 | VARCHAR2(40) mm/dd/yyyy HH24 format (Displays first hour of the day as hour 1 and last hour of the day as hour 00 of the following day) |
| Bal Operating Reserve Location Name | BOR_LOCATION_NAME | 4000.77 | VARCHAR2(30) |
| Bal Operating Reserve Location ID | BOR_LOCATION_ID | 4000.78 | NUMBER(*,0) |
| Location Type | BOR_LOCATION_TYPE | 4000.79 | VARCHAR2(100) (See Predetermined values below) |
| Bal Operating Reserve Region Name | BOR_REGION_NAME | 4000.80 | VARCHAR2(50) (See Predetermined values below) |
| DA Increment Offers (MWh) | DA_INCREMENT_OFFERS | 1370.16 | NUMBER |
| DA Operating Reserve Imports (MWh) | DA_OPRES_IMPORTS | 1370.17 | NUMBER |
| DA Internal Bilateral Purchases (MWh) | DA_INTERNAL_BILATERAL_PURCHASES | 1370.18 | NUMBER |
| DA Operating Reserve Injection (MWh) | DA_OPRES_INJECTION | 1370.19 | NUMBER |
| RT Operating Reserve Imports (MWh) | RT_OPRES_IMPORTS | 1375.20 | NUMBER |
| RT Internal Bilateral Purchases (MWh) | RT_INTERNAL_BILATERAL_PURCHASES | 1375.21 | NUMBER |
| RT Operating Reserve Injection (MWh) | RT_OPRES_INJECTION | 1375.22 | NUMBER |
| Operating Reserve Injection Deviation (MWh) | OPRES_INJECTION_DEVIATION | 1375.23 | NUMBER |
| DA Decrement Bids (MWh) | DA_DECREMENT_BIDS | 1370.24 | NUMBER |
| DA Demand Bids (MWh) | DA_DEMAND_BIDS | 1370.25 | NUMBER |
| DA Load Response Bids (MWh) | DA_LOAD_RESPONSE_BIDS | 1370.26 | NUMBER |
| DA Operating Reserve Exports (MWh) | DA_OPRES_EXPORTS | 1370.27 | NUMBER |
| DA Internal Bilateral Sales (MWh) | DA_INTERNAL_BILATERAL_SALES | 1370.28 | NUMBER |
| DA Operating Reserve Withdrawal (MWh) | DA_OPRES_WITHDRAWAL | 1370.29 | NUMBER |
| RT Load (MWh) | RT_LOAD | 3000.38 | NUMBER |
| Load Reconciliation Energy (MWh) | LOAD_RECONCILIATION_ENERGY | 3000.77 | NUMBER(22,3) |
| RT Operating Reserve Exports (MWh) | RT_OPRES_EXPORTS | 1375.30 | NUMBER |
| RT Internal Bilateral Sales (MWh) | RT_INTERNAL_BILATERAL_SALES | 1375.31 | NUMBER |
| RT Operating Reserve Withdrawal (MWh) | RT_OPRES_WITHDRAWAL | 1375.32 | NUMBER |
| Operating Reserve Withdrawal Deviation (MWh) | OPRES_WITHDRAWAL_DEVIATION | 1375.33 | NUMBER |
| Operating Reserve Generator Deviation (MWh) | OPRES_GENERATOR_DEVIATION | 1375.34 | NUMBER |
| Locational Total Deviation (MWh) | LOCATIONAL_TOTAL_DEVIATION | 1375.35 | NUMBER |
| Version | VERSION | 4000.07 | VARCHAR2(12) |

Possible Location Type Values:  ZONE, HUB, INTERFACE

Possible Bal Operating Reserve Region Name Values: East, West, RTO

# CSV Report Example

See Excel file titled “Operating Reserve Deviation Summary CSV Format.csv”

# XML Report Example

See XML file titled “Operating Reserve Deviation Summary XML Format.xml”

# Hyperlinks

This report contains the following hyperlinks:

| Column Name | Hyperlinked Report |
| --- | --- |
| DA Increment Offers (MWh) | DA Daily Energy Transactions |
| DA Operating Reserve Imports (MWh) | DA Daily Energy Transactions |
| DA Internal Bilateral Purchases (MWh) | DA Daily Energy Transactions |
| RT Operating Reserve Imports (MWh) | RT Daily Energy Transactions |
| RT Internal Bilateral Purchases (MWh) | RT Daily Energy Transactions |
| DA Decrement Bids (MWh) | DA Daily Energy Transactions |
| DA Demand Bids (MWh) | DA Daily Energy Transactions |
| DA Load Response Bids (MWh) | DA Daily Energy Transactions |
| DA Operating Reserve Exports (MWh) | DA Daily Energy Transactions |
| DA Internal Bilateral Sales (MWh) | DA Daily Energy Transactions |
| RT Load (MWh) | RT Daily Energy Transactions |
| RT Operating Reserve Exports (MWh) | RT Daily Energy Transactions |
| RT Internal Bilateral Sales (MWh) | RT Daily Energy Transactions |
| Operating Reserve Generator Deviation (MWh) | Operating Reserve Generator Deviations |

# Supporting Calculations

- DA Operating Reserve Injection (1370.19) = DA Increment Offers (1370.16) + DA Operating Reserve Imports (1370.17) + DA Internal Bilateral Purchases (1370.18)

- RT Operating Reserve Injection (1375.22) = RT Operating Reserve Imports (1375.20) + RT Internal Bilateral Purchases (1375.21)

- Operating Reserve Injection Deviation (1375.23) = ABS | (RT Operating Reserve Injection (1375.22) - DA Operating Reserve Injection (1370.19)

- DA Operating Reserve Withdrawal (1375.29) = DA Decrement Bids (1375.24) + DA Demand Bids (1375.25) + DA Load Response Bids (1375.26) + DA Operating Reserve Exports (1375.27) + DA Internal Bilateral Sales (1375.28)

- RT Operating Reserve Withdrawal (1375.32) = RT Load (3000.38) + Load Reconciliation Energy (3000.77) + RT Operating Reserve Exports (1375.30) + RT Internal Bilateral Sales (1375.31)

- Operating Reserve Withdrawal Deviation (1375.33) = ABS | (RT Operating Reserve Withdrawal (1375.32) - DA Operating Reserve Withdrawal (1375.29)

- Operating Reserve Generator Deviation (1375.34) = SUM(Supplier Netted Deviation MWh from Operating Reserve Generator Deviations report) for all supplier netted groups that fall within the transmission zone displayed.

  - Please note that the Supplier Netted Deviation MWh value appears on the Operating Reserve Generator Deviations report for each generator that belongs to the supplier netted group.  In order to properly recalculate the Operating Reserve Generator Deviation at the customer account-level, the Supplier Netted Deviation MWh should only be added once for each group, rather than once for each generator.

- Locational Total Deviation (1375.35) = Operating Reserve Injection Deviation (1375.23) + Operating Reserve Withdrawal Deviation (1375.33) + Operating Reserve Generator Deviation (1375.34)