MSRS Report Format Documentation

Regulation Credits

Version 2

Revision History

| Date | Revision | Description |
| --- | --- | --- |
| 10/19/2007 | 1 | Initial Distribution |
| 2/21/2020 | 2 | Removed all references to eSchedule(s) and/or eSchedule(s) reports |

# Report

MSRS Report Name:  Regulation Credits

Report short name for User Interface: Regulation Credits

Download File Name Abbreviation: RegCr

Data Granularity:  Hourly

Frequency:  Updated daily

Range Displayed on Report:  Start Date through End Date

# Supported Billing Line Items

- Regulation and Frequency Response Service Credit (2340)

# Report Content Summary

This report displays the customer account’s hourly Regulation Market Clearing Price Credit and Regulation Lost Opportunity Cost Credit where the customer’s RMCP Credit or Reg Lost Opportunity Cost Credit is not equal to 0.  The credits in this report do not reflect the customer account’s share of jointly owned units. All owners will see the full credit assigned to the unit.

# Summary of Changes and Special Logic

- The date range total row will only appear in the online version of the report.  It will not appear in the CSV and XML versions of the report.

- The Report will now display EPT and GMT Hour Ending.

- If the unit is a hydro unit, the Hydro Spill Indicator will indicate whether the unit was spilling or not for the given hour; If the unit is a non-hydro unit the Hydro Spill Indicator will display as null.

# Report Columns

The following columns will appear in the body of the report:

| Online and CSV Column Name | XML Column Name | Column Number | Data Type |
| --- | --- | --- | --- |
| Customer ID | CUSTOMER_ID | 4000.01 | INTEGER |
| Customer Code | CUSTOMER_CODE | 4000.02 | VARCHAR2(6) |
| EPT Hour Ending | EPT_HOUR_ENDING | 4000.05 | VARCHAR2(40) mm/dd/yyyy HH24 format (Displays first hour of the day as hour 1 and last hour of the day as hour 24) |
| GMT Hour Ending | GMT_HOUR_ENDING | 4000.06 | VARCHAR2(40) mm/dd/yyyy HH24 format (Displays first hour of the day as hour 1 and last hour of the day as hour 00 of the following day) |
| Unit ID | UNIT_ID | 4000.63 | NUMBER(8,0) |
| Unit Name | UNIT_NAME | 4000.64 | VARCHAR2(60) |
| Unit Ownership Share | UNIT_OWNERSHIP_SHARE | 3000.80 | NUMBER |
| PJM-Assigned Reg (MWh) | PJM_ASSIGNED_REG | 2340.17 | NUMBER(22,3) |
| Self-Scheduled Reg (MWh) | SELF_SCHEDULED_REG | 2340.18 | NUMBER(22,3) |
| RMCP ($/MWh) | RMCP | 3000.57 | NUMBER(10,2) |
| RMCP Credit ($) | RMCP_CREDIT | 2340.19 | NUMBER |
| Bias Factor | BIAS_FACTOR | 2340.20 | NUMBER |
| RT LMP Desired MWh | RT_LMP_DESIRED_MWH | 3000.35 | NUMBER(22,3) |
| RT Generator LMP ($/MWh) | RT_GENERATOR_LMP | 3000.25 | NUMBER(12,6) |
| Hydro Spill Indicator | HYDRO_SPILL_INDICATOR | 4000.67 | VARCHAR2(1) See possible values below |
| Reg Offer Price ($/MWh) | REG_OFFER_PRICE | 2340.21 | NUMBER(14,2) |
| Reg Offer Amount ($) | REG_OFFER_AMOUNT | 2340.22 | NUMBER |
| Regulation Lost Opportunity Cost ($) | REG_LOC | 2340.23 | NUMBER |
| Regulation Lost Opportunity Cost Credit ($) | REG_LOC_CREDIT | 2340.24 | NUMBER |
| Version | VERSION | 4000.07 | VARCHAR2(12) |

Possible Hydro Spill Indicator Values: Y or N

# CSV Report Example

See Excel file titled “Regulation Credits CSV Format.csv”

# XML Report Example

See XML file titled “Regulation Credits XML Format.xml”

# Hyperlinks

The online version of this report does not contain hyperlinks.

# Supporting Calculations

RMCP Credit (2340.19) = (PJM-Assigned Reg (2340.17) + Self-Scheduled Reg (2304.18)) * RMCP (3000.57)

Reg Offer Amount (2340.22) = PJM-Assigned Reg (2340.17) * Reg Offer Price (2340.21)

Regulation Lost Opportunity Cost Credit (2340.24) = MAX((Regulation Lost Opportunity Cost (2340.23) + Reg Offer Amount (2340.22) - (PJM-Assigned Reg (2340.17) * RMCP (3000.57)), 0)